| Length in bytes | BDF Header: | EDF Header: | Description |
| --- | --- | --- | --- |

| 8 bytes | Byte 1: "255" (non ascii) | Byte 1: "0" (ASCII) |  | Identification code |
| --- | --- | --- | --- | --- |
|  | Bytes 2-8 : "BIOSEMI" (ASCII) | Bytes 2-8 : " "(ASCII) |  |  |
| 80 bytes | User text input (ASCII) |  |  | Local subject identification |
| 80 bytes | User text input (ASCII) |  |  | Local recording identification |
| 8 bytes | dd.mm.yy (ASCII) |  |  | Startdate of recording |
| 8 bytes | hh.mm.ss (ASCII) |  |  | Starttime of recording |
| 8 bytes | (ASCII) |  |  | Number of bytes in header record |
| 44 bytes | "24BIT" (ASCII) |  | "BIOSEMI" (ASCII) | Version of data format. |
| 8 bytes | (ASCII) |  |  | Number of data records "-1" if unknown |
| 8 bytes | e.g.: "1" (ASCII) |  |  | Duration of a data record, in seconds |
| 4 bytes | e.g.: "257" or "128" (ASCII) |  |  | Number of channels (N) in data record |
| N x 16 bytes | e.g.: "Fp1", "Fpz", "Fp2", etc (ASCII) |  |  | Labels of the channels |
| N x 80 bytes | e.g.: "active electrode", "respiration belt" (ASCII) |  |  | Transducer type |
| N x 8 bytes | e.g.: "uV", "Ohm" (ASCII) |  |  | Physical dimension of channels |
| N x 8 bytes | e.g.: "-262144" (ASCII) |  | e.g.: "-32768" (ASCII) | Physical minimum in units of physical dimension |
| N x 8 bytes | e.g.: "262143" (ASCII) |  | e.g.: "32767" (ASCII) | Physical maximum in units of physical dimension |
| N x 8 bytes | e.g.: "-8388608" (ASCII) |  | e.g.: "-32768" (ASCII) | Digital minimum |
| N x 8 bytes | e.g.: "8388607" (ASCII) |  | e.g.: "32767" (ASCII) | Digital maximum |
| N x 80 bytes | e.g.: "HP:DC; LP:410" |  | e.g.: "HP:0,16; LP:500" | Prefiltering |
| N x 8 bytes | For example: "2048" (ASCII) |  |  | Number of samples in each data record. (Sample-rate if Duration of data record = "1"). |
| N x 32 bytes | (ASCII) |  |  | Reserved |